Cheng, Y., et al. (2025). "Determination and Application of a New Drought Resistant Control Water Level for Multi-Reservoir Systems Based on a Water Resource Allocation Model." Water Resources Management 39(13): 7227-7241. 10.1007/s11269-025-04294-4. https://www.scopus.com/pages/publications/105010600034?origin=resultslist.

With the increasing frequency of global drought events, implementing effective monitoring and early warning systems is essential. The drought-limited water level (DLWL) is a new indicator for drought management in China. This study proposes a refined optimization method for determining the DLWL and drought-resistant operational strategies for each reservoir in multi-reservoir systems based on a general water allocation and simulation model. This study takes representative reservoirs in the North China Plain as a case study. Results show that DLWL effectively mitigates regional water shortages during droughts, reducing overall shortages by 12% in light dry years and 27% in extremely dry years, and reducing severe damage from 75 to 25% by transforming it into more moderate wide and shallow damage during consecutive dry years. In addition, the refined method proved superior to the traditional aggregated approach by enabling more accurate determination of reservoir-specific DLWL, improving drought management efficiency, and mitigating water shortages. The method is well-suited for DLWL determination in complex multi-reservoir systems and provides important references for drought-resistant management of reservoirs. © The Author(s), under exclusive licence to Springer Nature B.V. 2025.

Li, S., et al. (2025). "Multi-objective optimization of seasonal emergency storage in cascade reservoirs for enhanced drought resilience and hydropower synergy." Journal of Water and Climate Change 16(10): 3046-3066. 10.2166/wcc.2025.816. https://www.scopus.com/pages/publications/105023556481?origin=resultslist.

Optimizing the seasonal emergency storage capacity (SESC) of cascade reservoirs is crucial for enhancing water supply and hydropower generation (HG), particularly in the face of extreme drought events. However, this task becomes exceptionally challenging due to the complex interplay of spatiotemporal drought variability and the topographically intricate nature of the basin. This study presents a multi-objective optimization framework for enhancing SESC in cascade reservoirs to simultaneously improve HG and drought resilience. Using a dynamic drawdown strategy, the model incorporates spatiotemporal drought variability and reservoir constraints. Applied to a five-reservoir cascade on the Yangtze River, the model outperforms the standard operation policy (SOP), yielding an 8.7% increase in hydropower output, a 45.5% reduction in river drought duration, and a 4.5% improvement in low-flow conditions. The framework offers robust decision-making support for climate-adaptive water resource management. © 2025 The Authors.

Li, S., et al. (2024). "Unlocking synergies of drawdown operation: Multi-objective optimization of reservoir emergency storage capacity." Journal of Environmental Management 368. 10.1016/j.jenvman.2024.122148. https://www.scopus.com/pages/publications/85201064481?origin=resultslist.

Optimizing reservoir drawdown operations holds significant implications for hydropower generation, water supply, and drought mitigation strategies. However, achieving multi-objective optimization in reservoir drawdown operations poses fundamental challenges, particularly considering emergency storage capacity and seasonal drought patterns. This study introduces a novel multi-objective optimization framework tailored for a mega reservoir, focusing on drawdown operations to enhance hydropower generation and water supply reliability. A drawdown operation model leveraging a multi-objective ant lion optimizer is developed to simultaneously maximize reservoir hydropower output and minimize water shortage rates. China's Three Gorges Reservoir (TGR), situated over the upper reaches of the Yangtze River, constitutes the case study, with the standard operation policy (SOP) serving as a benchmark. Results showcase the efficacy of the proposed method, with substantial improvements observed: a 10.6% increase in hydropower output, a 6.0% reduction in water shortage days, and a 9.5% decrease in minimal reservoir water release compared to SOP. This study provides robust technical and scientific bolster to optimize reservoir ESC and enhance the synergy between hydropower generation, water supply, and drought resilience. Additionally, it offers decision-makers actionable strategies that account for emergency water supply capacities. These strategies aim to support mega reservoir's resilience against extreme drought events facilitating the collaboration between modelers and policy-makers, by means of intelligent optimization and decision-making technologies. © 2024 Elsevier Ltd

Wan, W., et al. (2025). "Integrating Drought Warning Water Level With Analytical Hedging for Reservoir Water Supply Operation." Water Resources Research 61(8). 10.1029/2024WR038680. https://www.scopus.com/pages/publications/105012919631?origin=resultslist.

The increasing frequency of droughts worldwide, along with their substantial societal impacts, underscore the need for improved drought early warning and enhanced water supply security. While reservoir drought management strategies, including hedging theory and drought warning/limited water level (DWWL), have been extensively studied for their potential to alleviate water supply stress, effective control schemes remain lacking. This study proposes a framework to enhance reservoir drought resistance. The framework starts with the analytical derivation of the optimal hedging rule (HR) for reservoirs supplying multiple competing users, applicable to both real-time and long-term optimizations. DWWLs and their seasonal segmentation are then determined based on the optimal water supply process and multivariate time-series clustering. By utilizing reservoir water level and meteorological drought indicator to monitor drought conditions and characterize reservoir effective storage, a real-time operational strategy is ultimately proposed that integrates analytical HR with effective storage. We demonstrate the application in the Danjiangkou Reservoir in China, and the proposed operation is testified under both long-term periods and extreme dry conditions. Results indicate that the analytical HR outperforms conventional rule curves in nearly every aspect, including water supply benefits, reliability, and refill rate. The DWWL proves crucial during extreme drought conditions; when water levels fall below DWWL, the HR considering DWWL restricts water supply earlier, thereby preventing the reservoir from dropping dramatically to critically low levels. This proposed framework is transferable to other water supply reservoirs, offering scientific support, operational relevance, and guidance for drought mitigation and response. © 2025. The Author(s).